Update Axpert 5kVA Program SOP(DSP)

Step1: Connect the DC power or battery (must be more than start up voltage, i.e. >46V for 5kVA model) to the battery connector, then switch on the AXPERT.

Step2: Run the programming application “ReflashTool_Xseries.exe” by clicking Axpert 5k Reflash2013xxxx >> Reflash>> ReflashTool_Xseries.exe Before opening ReflashToll, make sure all application communication software using the COM port1 is closed. The application software include some UPS monitoring software.

Step3: Choose valid serial port which connect to inverter device

Step4: Click “Update ” button and will pop

click the button   ,then will pop

Step5: When the processing bar is up to 100%, a dialog will pop to remind you the programming is successful! Now the software of Inerter is updated completely!

Step6: Exit the reflash tool and turn off Axpert.

Step7:After updating the fireware,Please open the monitoring software(WatchPower) in your PC (you can install the WatchPower with a CD inside of the package when your received the unit) ,you should Restore to the defaults,then set the configuration you want .