Controller software-update

Step 1:

Download the programmer CLM3SProgrammer.exe for Windows operating system from http://www.epsolarpv.com/download/. No need for installation.

Download the updated software for controller:

DisplayXX-Verx.x. lof  for Controller Display part

ChargeXX-Verx.x. lof  for charge control part

XX represent the current:20A,30A,45A or 60A

Step 2:

Copy updated file to any directory, then double click  CLM3SProgrammer.exe  to running.

Diagram 1

Step 3:

Click ‘Browse’, select the software DisplayXX-Verx.x. lof or ChargeXX-Verx.x. lof.

When update both, there is no particular order.

Step 4:

Connect the serial port of the controller to PC SCI(serial communication interface) by cable，Connect power to the Battery input of the controller. Please don’t power on the controller at this moment.

Step 5:

Select commport from drop down list box. Click ‘Open’, then power on the controller.

Step 6：

When the progress bar come to end, there will display ‘Program Update Finished’, then disconnect the controller from PC and power off.

Step 7:

Recover the system connection.

Note:

- Before Click ‘Open’ button, the controller must be power off.

- If the update progress breaks unexpected, there will display “Connect Time Out”, Please power off the controller, and redo according to the above instructions.

- The controller will be damaged, if use unsuited software to update the controller.

- Baud of the PC SCI(serial communication interface) will be 115200bps.

- If use user’s cable, please make sure that it’s Cross cable.