- The procedures for updating the firmware on InfiniE 5.5KW

Step 1:  Connect the grid voltage to the AC INPUT connector.Then connect PC and inverter device with serial cable.Only through RS232 port update.

AC INPUT

RS232

- Step 2: Open the document “Reflash” and execute“ReflashTool.exe”

Note: UPSReflashTool.exe can only be executed in OS of windows

Monitor software like SolarPower.exe should be terminated.

- Step 3: Choose valid serial port which connect to inverter device

Step 4: Click “Update DSP” button and wait for 15 seconds to update dsp

- Step 5:  When the DSP has been updated successful, click “Update MCU” and wait for 15 seconds to update mcu

- Application Notes:

- If programming failed in Step4 or Step5, this may because of communication lost. Please close “UPSReflashTool.exe”.Then restart from Step 4 or Step 5.

- Note:The Serial port number can be found via Computer management>>Device manage>>Ports(COM & Port)